Signal Rescue Lab

Instructor answer key | Hamming(7,4) | Even parity

Use this key to check reasoning, not only final codewords. A correct syndrome without visible parity counts is not yet independent mastery.

## 1A  The impossible repair

The missing bit cannot be determined from the damaged string alone. Both 0 and 1 are possible unless the sender adds an agreed rule or redundancy.

## 1B  Even-parity sprint

| data | check bit | total 1s | audit |
| --- | --- | --- | --- |
| 101 | 0 | 2 | even |
| 111 | 1 | 4 | even |
| 000 | 0 | 0 | even |
| 0101 | 0 | 2 | even |

## 1C  Address detective

| position | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| check 1 | YES |  | YES |  | YES |  | YES |
| check 2 |  | YES | YES |  |  | YES | YES |
| check 4 |  |  |  | YES | YES | YES | YES |

Addresses: position 3 = 1+2; position 5 = 1+4; position 6 = 2+4; position 7 = 1+2+4.

Facilitator move: students should say which questions contain the position before adding the labels.

Seven-slot machine and warm-ups

Answer key page 2

## 2A  Roles and check groups

| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- |
| p1 | p2 | d1 | p4 | d2 | d3 | d4 |
| check 1 | check 2 | data 1 | check 4 | data 2 | data 3 | data 4 |

| position | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| check 1 | YES |  | YES |  | YES |  | YES |
| check 2 |  | YES | YES |  |  | YES | YES |
| check 4 |  |  |  | YES | YES | YES | YES |

## 2B  Training wheels: message 1 0 1

| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 1 | 0 | 1 | 0 |
| check 1 | check 2 | data 1 | check 4 | data 2 | data 3 | fixed 0 |

| check 1: 1+1+0+0=2 | check 2: 0+1+1+0=2 | check 4: 1+0+1+0=2 |
| --- | --- | --- |

Completed word: 1011010.

## 2C  Training wheels: message 0 1 1

| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 0 | 1 | 1 | 0 |
| check 1 | check 2 | data 1 | check 4 | data 2 | data 3 | fixed 0 |

Completed word: 1100110. Each parity group contains two 1s.

Terminology: the warm-up fixes position 7 at zero. It is a restricted subcode of Hamming(7,4), not the standard Hamming code.

Full Hamming(7,4) encoding

Answer key page 3

## 3A  Four-move routine

1 label positions 1-7; 2 place data in 3,5,6,7; 3 choose p1,p2,p4 for even parity; 4 verify all three checks.

## 3B  Message 1 0 1 1

| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| check 1 | check 2 | data 1 | check 4 | data 2 | data 3 | data 4 |

| check 1: 0+1+0+1=2 | check 2: 1+1+1+1=4 | check 4: 0+0+1+1=2 |
| --- | --- | --- |

Codeword: 0110011. Check bits: p1=0, p2=1, p4=0.

## 3C  Speed round

| message | codeword | check bits p1,p2,p4 |
| --- | --- | --- |
| 0000 | 0000000 | 0,0,0 |
| 1111 | 1111111 | 1,1,1 |
| 0101 | 0100101 | 0,1,0 |
| 1100 | 0111100 | 0,1,1 |

Common error: placing message bits consecutively in positions 1-4. Require the role row before any parity arithmetic.

Repair and decode

Answer key page 4

## 4A  Repair 0 1 1 0 0 0 1

| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 0 | 0 | 0 | 1 |
| check 1 | check 2 | data 1 | check 4 | data 2 | data 3 | data 4 |

| check | positions | 1s | result | label |
| --- | --- | --- | --- | --- |
| 1 | 1,3,5,7 | 2 | pass | 0 |
| 2 | 2,3,6,7 | 3 | fail | 2 |
| 4 | 4,5,6,7 | 1 | fail | 4 |

Syndrome = 2+4 = 6. Flip position 6. Corrected word = 0110011. Recovered message = 1011.

## 4B  Packet repair race

| packet | received | failed | corrected | message |
| --- | --- | --- | --- | --- |
| A | 0101101 | 4 | 0100101 | 0101 |
| B | 0111101 | 1+2+4=7 | 0111100 | 1100 |

No-error case: if all checks pass, syndrome 0 means leave the codeword unchanged. Under the single-error promise, any syndrome 1-7 tells which bit to flip.

Exchange, certification, and bonus

Answer key page 5

## 5A-5B  Partner exchange

Answers vary. A valid sender word has even parity in all three groups. The receiver's syndrome must equal the sender's secretly flipped position, and extraction after repair must reproduce the sender's four-bit message.

Extension: ask early finishers to flip a check bit (1,2,4) and compare with flipping a message bit (3,5,6,7). Both are corrected; only a data-bit error changes the extracted message before repair.

## 6A  Solo certification

| part | result | checks / syndrome | final message |
| --- | --- | --- | --- |
| A | 0011001 | p1=0, p2=0, p4=1 | 1001 |
| B | error at 5; corrected 0011001 | failed 1+4 | 1001 |

## 6B  Extended Hamming(8,4) decision table

| syndrome | overall parity | receiver action |
| --- | --- | --- |
| 0 | even | no error |
| nonzero | odd | correct one error in positions 1-7 |
| 0 | odd | flip overall parity bit 8 |
| nonzero | even | detect two errors; do not correct |

Session mastery criterion: the student can encode and repair the solo certification without a partner, while showing the three check calculations.

Source: R. W. Hamming, 'Error Detecting and Error Correcting Codes,' Bell System Technical Journal 29(2), 1950, pp. 147-160. https://doi.org/10.1002/j.1538-7305.1950.tb00463.x