WORKSHEET 1

# Mission Messages and Binary

| Name: __________________________________________ | Date: ____________________ |
| --- | --- |

1.  Read the four-flash language	EASY

In the mission codebook, red means 0 and green means 1. Use the reference codebook on the final page.

1. Decode each transmission:  0001 = ____________________     1000 = ____________________

2. Write the code for each report:  Salt detected = __________     Return to shelter = __________

3. Circle every pair that differs in exactly one position:   1010/1011     0000/1111     0100/0101

4. A probe sends 1100. One flash flips. Write all four possible received strings.

2.  How many messages fit?	EASY

Complete the doubling table.

| Number of flashes n | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| Possible strings 2ⁿ | 2 |  |  |  |  |

Why are three flashes insufficient for sixteen reports?

Why are four flashes exactly enough?

3.  Distance measures danger	EASY

The Hamming distance is the number of positions where two strings differ.

a)  d(1010, 1011) = ______     d(0000, 1110) = ______     d(0110, 1001) = ______

b) Which pair above is most easily confused by one flipped bit? Explain.

c) Mark the differing positions: 1 0 0 1 and 1 1 1 0. Distance = ______

6.  Build and defend a code	CHALLENGE

Choose three valid five-bit messages. Keep every pair at distance at least 3.

| Message | Five-bit code | Why it is far enough from the others |
| --- | --- | --- |
| A |  |  |
| B |  |  |
| C |  |  |

For each message, write one string produced by flipping a single bit. Check that none of your examples could be interpreted as a one-bit error from two different messages.

WORKSHEET 2

# Detect Repeat or Repair

| Name: __________________________________________ | Date: ____________________ |
| --- | --- |

1.  The parity watchdog	EASY

Append one parity bit so the total number of 1s is even.

| Four data bits | Parity bit | Five transmitted bits |
| --- | --- | --- |
| 1010 |  |  |
| 1110 |  |  |
| 0001 |  |  |
| 1101 |  |  |

a) The receiver gets 10101. Is its total parity even or odd? ____________________

b) What can the watchdog conclude?

c) Can it identify which position changed? Explain.

3.  Repeat the whole message	EASY

The original four-bit report is sent three times.

| Copy | Bit 1 | Bit 2 | Bit 3 | Bit 4 |
| --- | --- | --- | --- | --- |
| 1 - original message | 1 | 0 | 1 | 0 |
| 2 | 1 | 0 | 1 | 1 |
| 3 | 0 | 0 | 1 | 0 |
| Column majority |  |  |  |  |

Recovered message: ____________________. Use the codebook on the final page.

Decoded report: ________________________________________________________________

How many transmitted bits were used? ________     How many were extra copies? ________

4.  When majority voting fails	CHALLENGE

Create three copies of one four-bit message so that two errors in the same column make majority voting return the wrong bit. Show the original and all three received copies.

Original: __________     Copy 1: __________     Copy 2: __________     Copy 3: __________

State precisely what triple repetition guarantees and what it does not guarantee.

WORKSHEET 2 CONTINUED

# Detect Repeat or Repair

6.  Communication budget	CHALLENGE

A probe can transmit at most 70 bits during a short communication window.

a) How many four-bit reports fit with no protection?

b) How many fit if every report is repeated three times?

c) How many fit using Hamming(7,4)?

d) Which method would you choose if single-bit correction is required? Defend the tradeoff.

7.  Burst-error thought experiment	CHALLENGE

A radiation burst flips three adjacent transmitted bits. Compare two layouts:

Layout A sends each three-copy message consecutively.

Layout B interleaves bits from many different messages.

Which layout is more likely to spread the damage across different protected blocks? Why can spreading errors help?

WORKSHEET 3

# Hamming(7,4) Inspect and Repair

| Name: __________________________________________ | Date: ____________________ |
| --- | --- |

1.  Give every position an address	EASY

Write the inspector answers in the order Inspector 4, Inspector 2, Inspector 1.

| Position | Binary address | Inspector 1? | Inspector 2? | Inspector 4? |
| --- | --- | --- | --- | --- |
| 1 | 001 | yes | no | no |
| 2 | 010 | no | yes | no |
| 3 | 011 |  |  |  |
| 4 | 100 |  |  |  |
| 5 | 101 |  |  |  |
| 6 | 110 |  |  |  |
| 7 | 111 |  |  |  |

Why does 000 mean “no error”?

Why are positions 1, 2, and 4 natural places for the check bits?

2.  Place data and checks	EASY

Fill the roles in order. Use P for a parity/check bit and D for a data bit.

| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- |

Complete the watcher sets:

Inspector 1 watches ____________________; Inspector 2 watches ____________________; Inspector 4 watches ____________________.

3.  Encode report 1010	EASY

Put data 1010 into positions 3, 5, 6, and 7. Choose P1, P2, and P4 so every inspector sees an even number of 1s.

| P1 | P2 | D1 | P4 | D2 | D3 | D4 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 |  | 0 | 1 | 0 |

Inspector calculations: P1 = ________; P2 = ________; P4 = ________.

Seven-bit codeword: ________________________________.

WORKSHEET 3 CONTINUED

# Hamming(7,4) Inspect and Repair

4.  Locate one damaged bit	EASY

Received word: 1011000. Test even parity.

| Inspector | Positions checked | Number of 1s | Satisfied or complains? |
| --- | --- | --- | --- |
| 1 | 1, 3, 5, 7 |  |  |
| 2 | 2, 3, 6, 7 |  |  |
| 4 | 4, 5, 6, 7 |  |  |

Add the labels of complaining inspectors: ____________________. Error position: ________.

Flip that bit. Repaired codeword: ____________________. Extract positions 3, 5, 6, 7: ____________________.

Use the codebook on the final page. Report: ________________________________________________.

WORKSHEET 4

# Limits Noise and Research

| Name: __________________________________________ | Date: ____________________ |
| --- | --- |

1.  Count each error neighborhood	EASY

For an n-bit protected message, the receiver may see the original or any one of n single-bit flips. Each correctable message therefore needs n + 1 exclusive received strings.

| n | All strings 2ⁿ | Slots per message n + 1 | 16(n + 1) needed | Can 16 fit? |
| --- | --- | --- | --- | --- |
| 5 | 32 | 6 |  |  |
| 6 | 64 | 7 |  |  |
| 7 | 128 | 8 |  |  |

Why can no clever rearrangement make five or six bits sufficient?

What is special about 16(7 + 1) = 2⁷?

3.  Transmission cost	EASY

Hamming(7,4) converts every four data bits into seven transmitted bits.

a) Extra bits per four-bit message: ____________________

b) Transmitted bits for one 8-bit byte: ____________________

c) The complete text used in the slides has 3,359,405 bytes. Confirm that encoding it uses 47,031,670 transmitted bits. Show the multiplication.

d) Compare with repeating every bit three times. How many transmitted bits would that require?

WORKSHEET 4 CONTINUED

# Limits Noise and Research

5.  Predict recovery with probability	CHALLENGE

For independent flip probability p, a seven-bit codeword has no errors with probability (1 - p)⁷ and exactly one error with probability 7p(1 - p)⁶.

1. Write the probability that Hamming(7,4) is within its guarantee:

P(0 or 1) =

2. Evaluate it for p = 0.03.

3. A byte uses two codewords. Estimate the probability that both are within the guarantee by squaring your answer.

4. Repeat for p = 0.10. What changes most sharply?

7.  Design a research test	CHALLENGE

Choose one question.

How does exact text recovery change as the flip rate moves from 0 to 20 percent?

Can a different seven-bit code protect more than sixteen four-bit reports?

How should a code change when two errors must be corrected?

How does interleaving change damage from a fixed-length burst?

Claim or prediction:

Variables and test procedure:

Evidence to record:

What result would change your mind?

WORKSHEET REFERENCE

# Original NILA Mission Codebook

The sixteen four-bit reports used throughout the worksheets.

| Code | Mission report | Code | Mission report |
| --- | --- | --- | --- |
| 0000 | Systems normal | 1000 | Liquid water detected |
| 0001 | Battery low | 1001 | Organic molecules detected |
| 0010 | Camera ready | 1010 | Possible biological pattern |
| 0011 | Camera damaged | 1011 | Mission emergency |
| 0100 | Drill ready | 1100 | Radiation spike |
| 0101 | Drill jammed | 1101 | Surface crack opening |
| 0110 | Ice sample collected | 1110 | Warm plume detected |
| 0111 | Salt detected | 1111 | Return to shelter |

Hamming(7,4) reference

Check positions: 1, 2, 4     Data positions: 3, 5, 6, 7

Inspector 1 watches 1, 3, 5, 7. Inspector 2 watches 2, 3, 6, 7. Inspector 4 watches 4, 5, 6, 7.

Counting bound

If M length-n codewords must correct every one-bit error, their radius-one neighborhoods cannot overlap, so M(n + 1) ≤ 2ⁿ. Equality for M = 16 and n = 7 makes Hamming(7,4) a perfect code.